Embedding it better...

µTasker Document

- I.MX RT Refefence File System Layout in SPI Flash

Table of Contents

1. Introduction	3

2. File Locations	3

# Introduction

The i.MX RT reference configuration assumes that the device has 8MBytes of external SPI Flash (or more)

The file system has been positioned to start after a 64k boot loader, 64k serial loader, and 512k application. In addition a 64k parameter system is located after the file system's end. The SPI flash is assumed to have a granularity of 4k. The QSPI FLASH characteristics allow singe blocks (sectors) to be deleted and bytes to be programmed – cumulative programming of individual bits in a bytes is possible.

The resulting file system size if 7.376MBytes.

Note that some i.MX Rt boards use HyperFlash, which is however less suitable for file system operation due to its large sector size of 256k. This file system layout can not be used on such a board!

112k file size

This allows up to 62 files of each up to 112k (0x1c000) in size (- 2/4 bytes for length plus optionally 1 byte for MIME type). The file at location ‘z’ can accept a file of up to 560k and is generally reserved for firmware upload space (allowing 512k code to be securely loaded).

# File Locations

| Address of block | Web page with available space | Block number and actual size of file |
| --- | --- | --- |
| 0x600a0000 Block 0 File ‘0’ | 112k 0Menu.htm | 0 <112k |
| 0x600bc000 Block 1 File ‘1’ | 112k 1Lan.htm | 1 <112k |
| 0x600d8000 Block 2 File ‘2’ | 112k 2I_O.htm | 2 <112k |
| 0x600f4000 Block 3 File ‘3’ | 112k 3admin.htm | 3 <112k |
| 0x60110000 Block 4 File ‘4’ | 112k 4EmailSetup.htm | 4 <112k |
| 0x6012c000 Block 5 File ‘5’ | 112k 5serial.htm | 5 <112k |
| 0x60148000 Block 6 File ‘6’ | 112k 6help.htm | 6 <112k |
| 0x60164000 Block 7 File ‘7’ | 112k 7dynamic.htm | 7 <112k |
| 0x60180000 Block 8 File ‘8’ | 112k 8Stats.htm | 8 <112k |
| 0x6019c000 Block 9 File ‘9’ | 112k 9Generate.htm | 9 - 5 bytes |
| 0x601b8000 Block 10 File ‘A’ | 112k ALCD.htm | 10 - <112k |
| 0x601d4000 Block 11 File ‘B’ | 112k BFTP_client.htm | 11 - <112k |
| 0x601f0000 Block 12 File ‘C’ | 112k – free file space | 12 |
| 0x6020c000 Block 13 File ‘D’ | 112k Dback.jpg | 13 <112k |
| 0x60228000 Block 14 File ‘E’ | 112k Eboard.jpg | 14 <112k |
| 0x60244000 Block 15 File ‘F’ | 112k – free file space | 15 |
| 0x60260000 Block 16 File ‘G’ | 112k – free file space | 16 |
| 0x6027c000 Block 17 File ‘H’ | 112k – free file space | 17 |
| 0x60298000 Block 18 File ‘I’ | 112k – free file space | 18 |
| 0x602b4000 Block 19 File ‘J’ | 112k – free file space | 19 |
| 0x602d0000 Block 20 File ‘K’ | 112k – free file space | 20 |
| 0x602ec000 Block 21 File ‘L’ | 112k – free file space | 21 |
| 0x60308000 Block 22 File ‘M’ | 112k – free file space | 22 |
| 0x60324000 Block 23 File ‘N’ | 112k – free file space | 23 |
| 0x60340000 Block 24 File ‘O’ | 112k – free file space | 24 |
| 0x6035c000 Block 25 File ‘P’ | 112k – free file space | 25 |
| 0x60378000 Block 26 File ‘Q’ | 112k – free file space | 26 |
| 0x60394000 Block 27 File ‘R’ | 112k – free file space | 27 |
| 0x603b0000 Block 28 File ‘S’ | 112k – free file space | 28 |
| 0x603cc000 Block 29 File ‘T’ | 112k – free file space | 29 |
| 0x603e8000 Block 30 File ‘U’ | 112k – free file space | 30 |
| 0x60404000 Block 31 File ‘V’ | 112k – free file space | 31 |
| 0x60420000 Block 32 File ‘W’ | 112k – free file space | 32 |
| 0x6043c000 Block 33 File ‘X’ | 112k – free file space | 33 |
| 0x60458000 Block 34 File ‘Y’ | 112k – free file space | 34 |
| 0x60474000 Block 35 File ‘Z’ | 112k – free file space | 35 |
| 0x60490000 Block 36 File ‘a’ | 112k aLogo.gif (2.8M space) | 36 |
| 0x604ac000 Block 37 File ‘b’ | 112k | 37 |
| 0x604c8000 Block 38 File ‘c’ | 112k | 38 |
| 0x604e4000 Block 39 File ‘d’ | 112k | 39 |
| 0x60500000 Block 40 File ‘e’ | 112k | 40 |
| 0x6051c000 Block 41 File ‘f’ | 112k | 41 |
| 0x60538000 Block 42 File ‘g’ | 112k | 42 |
| 0x60554000 Block 43 File ‘h’ | 112k | 43 |
| 0x60570000 Block 44 File ‘i’ | 112k | 44 |
| 0x6058c000 Block 45 File ‘j’ | 112k | 45 |
| 0x605a8000 Block 46 File ‘k’ | 112k | 46 |
| 0x605c4000 Block 47 File ‘l’ | 112k | 47 |
| 0x605e0000 Block 48 File ‘m’ | 112k | 48 |
| 0x605fc000 Block 49 File ‘n’ | 112k | 49 |
| 0x60618000 Block 50 File ‘o’ | 112k | 50 |
| 0x60634000 Block 51 File ‘p’ | 112k | 51 |
| 0x60650000 Block 52 File ‘q’ | 112k | 52 |
| 0x6066c000 Block 53 File ‘r’ | 112k | 53 |
| 0x60688000 Block 54 File ‘s’ | 112k | 54 |
| 0x606a4000 Block 55 File ‘t’ | 112k | 55 |
| 0x606c0000 Block 56 File ‘u’ | 112k | 56 |
| 0x606dc000 Block 57 File ‘v’ | 112k | 57 |
| 0x606f8000 Block 58 File ‘w’ | 112k | 58 |
| 0x60714000 Block 59 File ‘x’ | 112k | 59 |
| 0x60730000 Block 60 File ‘y’ | 112k | 60 |
| 0x6074c000 Block 61 File ‘z’ | 112k (max. 784k) | 61 |
| 0x60768000 Block 62 | 112k | 62 |
| 0x60784000 Block 63 | 112k | 63 |
| 0x607a0000 Block 64 | 112k | 64 |
| 0x607bc000 Block 65 | 112k | 65 |
| 0x607d8000 | 96k gap | - |
| 0x607f0000 Parameter 1 | 32k | Swap block 1 |
| 0x607f8000 Parameter 2 | 32k | Swap block 2 |

The set up to achieve this structure (in app_hw_iMX.h) is:

#define PARAMETER_BLOCK_START (FLEXSPI_FLASH_BASE + 0x7f0000) // SPI FLASH location

#define uFILE_START (FLEXSPI_FLASH_BASE + 0xa0000)            // SPI FLASH location

#define FILE_GRANULARITY (112 * 1024)               // each file a multiple of 112k

#define FILE_SYSTEM_SIZE (0x734000)        // 7.488MBytes reserved for file system

When a file system larger that 64k is used, the typedef for MAX_FILE_SYSTEM_OFFSET must be set to an unsigned long rather than unsigned short, which is adequate for smaller ones.

Also MAX_FILE_LENGTH must be defined as unsigned long if individual files can be of more than 64k in size.

These are both modified in types.h. This is however the default setting in iMX projects.

typedef unsigned long   MAX_FILE_LENGTH;        // over 64k file lengths

typedef unsigned long   MAX_FILE_SYSTEM_OFFSET; // offsets of over 64k in file system - use for file system sizes of larger than 64k

Modifications:
- V1.0 21.02.2020 First version